11/18/1996

SCO UNIX Driver LLI

RTFSCO Installation

==================================================

The following files will be used in this installation:

Files          Contents

-----------    -----------------------------------------

driver.o       RTFSCO Driver Program

install        Transfer dos format to unix format setup program

info           Used by netconfig

init           Used by netconfig

master         Used by netconfig

node           Used by netconfig

reconf         Used by netconfig

remove         Used by netconfig

space.c        Used by netconfig

system         Used by netconfig

1. Login as root user. (On maintenance mode)

2.

Installation Diskette in DOS format.

Put RTL SCO UNIX Driver diskette to floppy A: and use doscp

command to copy RTL SCO UNIX Driver into UNIX's directories.

# cd /

# doscp A:/sco/install /install

# chmod +x /install

# ./install

4.     Use netconfig utility

Avaiable options:

# netconfig

1) Add a chain

2) Remove a chain

3) Reconfigure an element in a chain

4) Quit                                          Select 1. Add a chain

5. Select top level of chain to Add or q to quit:       4. sco_tcp

(SCO TCP/IP for UNIX)

6. Select next level of chain to Add or q to quit:      Select RTL PCI Family

Fast Ethernet Driver

7. Add chain sco_tcp -> r5e0 (y/n)                      board 0 type 'y'

8. Enter the internet Address of this interface         n.n.n.n

9. Enter the netmask for this interface                 (default 255.255.255.0)

10. Does the interface use a broadcast address of all 1's (y/n)  default : y

11. Enter the boardcast address for this interface       default: n.n.n.n

12. Are these values correct ? (y/n)                    y

13. Pseudo ttys are currently configured, do you want to:

1) Add Pseudo ttys

2) Remove Pseudo ttys

Select an option or enter q to quit [q]          q

14.Available options:

1) Add a chain

2) Remove a chain

3) Reconfigure a element in a chain

q. Quit                                          Select option: q

15. Do you want to relink to kernel now ?               y

16. Do you want this kernel to boot by default (y/n)    y

17. Do you want the kernel environment rebuilt (y/n)    y

18. Reboot unix

# shutdown -y -g0